COLLEGE OF ENGINEERING AND TECHNOLOGY

SCHOOL OF BIOENGINEERING, DEPARTMENT OF CHEMICAL ENGINEERING

B. Tech. Open Elective

ACADEMIC YEAR 2023-24 – ODD SEMESTER

Continuous Learning Assessment III                      		 Set 4

| Reg. No. | R | A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Course Code: 18CHO104T | Course Title: PROCESS PLANT SAFETY |  |  |
| --- | --- | --- | --- |
| Sem & Year: V & III year | Date: 07/11/2023 | Duration: 100 Minutes | Max. Marks: 50 |

|  | Course Outcomes (COs) | Program Outcomes (POs) |  |  |  |  |  |  |  |  |  |  |  |  | PSOs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 1 |  | 2 | 3 |
| CO4 | Understand the various concepts of Hazard identification techniques | 2 | 3 | - | 2 | - | - | - | - | - | - | - | - | 3 |  | 1 | - |
| CO5 | Understand the various aspects of Occupational Health hazards, Safety legislation | 1 | - | 2 | - | - | - | - | - | - | - | - | 1 | 3 |  | - | 2 |

Part A Answer the Following     			10×1 Marks = 10 Marks

| Q. No. | Questions | Marks | CO | BL | PO/PSO | Marks Scored |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Which of the following special purpose footwear should you wear to avoid a buildup of static electricity? - Safety toe shoes - Electrically conductive shoes - Foundry shoes - Metatarsal guards | 1 | 5 | L2 | 3,2 |  |
| 2. | --------------- hard hat protects you from 20000 volts electric current - Class A - Class B - Class C - Bump caps | 1 | 5 | L1 | 3,2 |  |
| 3. | Which of the following types of gloves should you wear when working with chemicals? - Butyl rubber gloves - Fabric gloves - Leather gloves - Butyl and leather gloves | 1 | 5 | L2 | 3,2 |  |
| 4. |  | 1 | 5 | L1 | 3,2 |  |
| 5. | To emerge from Bhopal gas tragedy --------------- is enacted? a) Factories act b) environmental protection act c) Indian boiler’s act d) Mine’s act | 1 | 5 | L1 | 3,2 |  |
| 6. | Which is the way of determining risks and dangers in workplace? - Hazard analysis b) risk assessment c) fault tree analysis d) human error analysis | 1 | 4 | L2 | 1,3 |  |
| 7. | -------- method is used to evaluate the potential risks from a process and also to assess the potential loss is assessed - Fault tree analysis - HAZOP study c) Dow fire and explosion index d) Risk assessment | 1 | 4 | L1 | 1,3 |  |
| 8. | The guide word ‘NO or NOT’ in hazop study means - Quantitative increase - Quantitative decrease - Complete negation of the intentions - Complete substitution | 1 | 4 | L2 | 1,3 |  |
| 9. | If the fire and explosion index within 97-127 range, then the degree of hazard is - Light b) Moderate c) Intermediate d) Heavy | 1 | 4 | L1 | 1,3 |  |
| 10. | Calculate the risk from the risk table: Hazard; Likelihood; Severity; Risk. A; 5; 6; ?. - 20 b)15 c) 11 d) 30 | 1 | 4 | L2 | 1,3 |  |

| Q.No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ans. |  |  |  |  |  |  |  |  |  |  |

| Reg. No. | R | A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Part B  Answer the following      		4 x 4 Marks = 16 Marks

| Q. No. | Questions |
| --- | --- |
| 11. | Explain a note on Preliminary Hazard analysis (PHA). - Conducted to identify potential hazards and prioritize them according to the : - probability of an accident or injury being caused by the hazard. - severity of injury, illness or property damage that could result if the hazard caused an accident. - PHA can serves 2 purposes : - it can expedite bringing the new system on line, but at a substantially reduced risk of injuring workers. - it can serve as a guide for a future detailed analysis. - Experience and related expertise are important factors in conducting a preliminary review. |
| 12. | What is HAZOP? Discuss its basic principles. - A hazard and operability study is a procedure for the systematic, critical, examination of the operability of a process - When applied to a process design or an operating plant, it indicates potential hazards that may arise from deviations from the intended design conditions - A formal operability study is the systematic study of the design, vessel by vessel and line by line using ‘ guide words’ to help generate thoughts about the way deviations from the intended operating conditions can cause hazardous situations |
| 13. | Discuss on any two acts enacted for safety and health. |
| 14. | What is an occupational hazard? Explain on any one type. |

Part C	Answer the Following                     	2 x 12 Marks = 24 Marks

| Q. No. | Questions |
| --- | --- |
| 15a. | Describe the procedure of Fault tree analysis by constructing a fault tree. - Fault tree is the logical model of the relationship of the undesired event to more basic events. - The top event of the Fault tree is the undesired event. - The middle events are intermediate events - The basic events are at the bottom. - The logic relationship of events are shown by logic symbols or gates. Procedure: - Define the primary failure to be analyzed in other words identify the undesirable top event - Identify first level contributors which are just below the top level using the available technical information - Link these contributors to top level event by using logical gates (AND, OR gates), and also see the relationship, so that it will help to identify the appropriate logical gate - Identify the second level contributors and link to top by using logical gates. - Identify minimal cut set - Repeat the same steps till the basic causes - Finally complete and evaluate the FTA - Calculate probability of lowest level elements occurrence and also measure the probabilities from bottom up |
|  | OR |
| 15b. | Describe the Failure mode and Effects analysis procedure. FMEA Procedure - Determine failure mode for each process input. - For each failure mode, determine effects – Select a severity level for each effect. - Identify potential causes of each failure mode – Select an occurrence level for each cause. - List current controls for each cause – Select a detection for each cause. - Calculate the Risk Priority Number (RPN). - Develop recommended action , assign responsible person and take actions. - Assign the predicted Severity, Occurrence and Detection levels and compare RPNs Risk Priority number (rpn) - Severity× Occurrence × Detection = RPN - 1000 is maximum and 75* is considered OK!!! - Severity (S) - Importance of the effects on customer requirements. - 1 = Not sever, 10 = very sever - Occurrence (O) - Frequency with which a given cause occurs and creates failure modes. - 1 = NOT Likely, 10 = Very Likely - Detection (D) - The ability of the current control scheme to detect then prevent a given causes. 1 = Easy to Detect, 10 = Not easy to Detect Rating Scales - There are a wide variety of scoring anchors. - Two types of scales are 1-5 or 1-10. - The 1-5 scale makes it easier for the teams to decide on scores. - The 1-10 scale may allow for better precision in estimates and a wide variation in scores. - Zero(0) Ranking not allowed for RPN Rating Scales. |
| 16a. | Discuss the following for Eye and face protection: - Potential hazards - Control of hazards - Care and maintenance - List of PPEs Eye and Face Protection (Explain it all) Types of eye/face hazards - Impact - Heat - Chemicals - Dust - Light and/or Radiation TYPES OF EYES AND FACE PROTECTIVE EQUIPMENT - Safety Glass - Goggles - Face Shields - Welding Helmets - Absorptive Lenses Elimination or Control of Hazards - Machine Guards - Many types of machines such as lathes, grinders, and sanders are equipped with guards, shields and screens - Work Area Barriers - Operations such as sanding, grinding, welding, and lathe operations produce dust, vapors, and flying particles. - Ventilation - Ventilation, along with damping systems, can significantly reduce the amount of airborne particles that could be hazardous to your eyes |
|  | OR |
| 16b. | Explain a brief role on respiratory protective equipment. |

% of CO attainment is above 75% - Level 3           % of CO attainment is 60% -75% - Level 2

% of CO attainment is 50% -60% - Level 1 	       % of CO attainment is below 50% - Level 0

|  | Quality | Marks Scored | % of Marks | Attainment |
| --- | --- | --- | --- | --- |
| CO4 | L1 = 2 Marks |  |  |  |
|  | L2 = 11 Marks |  |  |  |
|  | L3 = 24 Marks |  |  |  |
| CO5 | L1 = 7 Marks |  |  |  |
|  | L2 = 30 Marks |  |  |  |
| POs/PSOs: | 1,2,3 |  |  |  |

| Reg. No. | R | A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Course Code: 18CHO104T | Course Title: PROCESS PLANT SAFETY |  |  |
| --- | --- | --- | --- |
| Sem & Year: V & III year | Date: 07/11/2023 | Duration: 5 Minutes | Max. Marks: 5 |

Continuous Learning Assessment IV

Match the following (Quiz component) (5x1 = 5 marks)

| 1. | Factories act | a. | Logical diagram |
| --- | --- | --- | --- |
| 2. | Gas mask | b. | Health and safety |
| 3. | Fault tree analysis | c. | PID diagram |
| 4. | Poor illumination | d. | Filters 0.1 - 2 % concentration |
| 5. | HAZOP | e. | Lacrimation |

| Q.No. | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Ans |  |  |  |  |  |